Paneer Packaging: Current Trends, Technologies, and Future Perspectives — A Comprehensive Review

Jyoti1; Atul K. Gupta2*; Bhuvnesh Kumar3

1Department of Food Science and Technology, Sharda University, Greater Noida, Uttar Pradesh, India.

2Sharda Research and Analytical Foundation, Sharda University, Greater Noida, Uttar Pradesh, India.

4Research and Development Cell, Sharda University, Greater Noida, Uttar Pradesh, India.

*Corresponding Author: atul.gupta2@sharda.ac.in

Abstract

A popular fresh acid-coagulated cheese in South Asia, paneer is a highly perishable dairy product that is prone to quick microbiological deterioration and physicochemical changes after production. The short shelf life of paneer under typical refrigeration conditions (2–4 days at 4–8°C) poses significant distribution and marketing challenges for the dairy industry. Packaging technology is the most practical and cost-effective technique to extend shelf life, preserve sensory attributes, and ensure the microbiological safety of paneer. This review covers both conventional approaches like vacuum packaging and polyethene pouches as well as state-of-the-art methods like modified atmosphere packaging (MAP), active packaging systems, nanocomposite and bio-nanocomposite films, edible coatings, and intelligent packaging technologies.

The literature from 2020 to 2026, which highlights developments in compostable multilayer laminates, cellulose nanocrystal-based packaging, plant-derived antimicrobial coatings, graphene oxide-reinforced films, and blockchain-integrated cold-chain monitoring, is given special attention. Along with pertinent international and Indian regulatory frameworks, methods for assessing physicochemical, microbiological, and sensory quality are thoroughly investigated. Environmental sustainability, consumer acceptance, and emerging trends are all discussed. Future directions and research gaps are identified to guide the development of safe, sustainable, and commercially viable paneer packaging options.

Keywords: Paneer; shelf life; modified atmosphere packaging; active packaging; nanocomposite films; dairy packaging; edible coatings; smart packaging; cellulose nanocrystals; biopolymer packaging; cold-chain monitoring

# 1. Introduction

An unripened fresh cheese coagulated by heat and acid, paneer holds a special place in the food and culture of the Indian subcontinent. It is made by coagulating warm cow or buffalo milk with acidulants such whey, lactic acid, or citric acid, then draining the whey, compacting the curd into a block, and submerging it in cold water [1,2]. The product is highly susceptible to microbial deterioration due to its firm, smooth texture, mild, milky flavour, and high moisture content (50-60%).

With over 230 million tonnes of milk produced annually, India is the largest dairy industry in the world [3]. Paneer makes up a sizable and quickly expanding portion of value-added dairy products. According to estimates, India produces more than 600,000 tonnes of paneer annually, and demand is rising as a result of the country's fast urbanisation, dietary changes toward plant-based meals high in protein, and growing food service sector [4,5]. The post-COVID-19 surge in organized retail and e-grocery platforms has boosted consumer demand for hygienically packed, longer-shelf-life paneer, further pressuring the industry to move beyond traditional packaging designs [6,7].

The short shelf life of paneer severely restricts its commercial viability. Unpackaged paneer deteriorates in 48–72 hours under normal refrigeration (4–8°C) because to oxidative rancidity, whey exudation, and the growth of psychrotrophic bacteria, molds, and yeasts [8,9]. Packaging is widely recognized as the most useful and financially feasible primary intervention for extending the shelf life of fresh dairy products. Paneer packaging needs to function as a mechanical barrier to avoid physical damage, a biological barrier to prevent microbial contamination, and a chemical barrier to stop the spread of moisture and oxygen [10,11].

Over the past five years, paneer packaging research has seen significant change. More recent studies (2020–2026) have concentrated on bio-nanocomposite films, active packaging based on plant essential oils, cellulose nanostructure reinforcements, freshness prediction aided by machine learning, and the integration of IoT sensors with cold-chain management platforms. Previous research focused on MAP, vacuum packaging, and polyethene pouches (12,13,14,15,16). No comprehensive analysis has collated developments in the full variety of packaging solutions utilized with paneer, despite this growing body of research. The current analysis addresses all relevant technological, regulatory, sustainable, and consumer factors by thoroughly examining published information from 1990 to 2026, with an emphasis on developments documented between 2020 and 2026.

2. Composition and Perishability of Paneer

## 2.1 Chemical Composition

The chemical composition of paneer is influenced by the type of milk (buffalo vs. cow), acidulant used, processing temperature, pressing time, and post-pressing water wash conditions. Table 1 compiles the proximate composition data for full-fat paneer that has been documented in the literature. Paneer's virtually neutral pH, water activity (aw > 0.96), relatively high moisture content (>52%), and nutrient-dense matrix of proteins, lipids, and residual lactose make it an ideal substrate for microbial development [1,2]. The casein-dominated protein fraction (~80% of total protein) is also susceptible to proteolytic breakdown by spoilage bacteria, which results in softening of texture, disagreeable off-flavors, and surface sliminess [17,18].

Table 1. Proximate Composition of Full-Fat Paneer Prepared from Buffalo and Cow Milk

| Parameter | Buffalo Milk Paneer | Cow Milk Paneer | References |
| --- | --- | --- | --- |
| Moisture (%) | 52.4–57.6 | 55.0–60.2 | [1] |
| Protein (%) | 19.6–22.4 | 16.8–20.1 | [2] |
| Fat (%) | 21.5–25.0 | 18.2–22.5 | [8] |
| Ash (%) | 1.7–2.1 | 1.5–1.9 | [9] |
| Carbohydrates (%) | 1.5–3.0 | 1.8–3.4 | [4] |
| pH | 5.4–5.7 | 5.5–5.8 | [1] |
| Water Activity (aw) | 0.96–0.98 | 0.97–0.99 | [10] |

## 2.2 Microbial Spoilage and Shelf Life

Microbial decomposition is the primary factor shortening paneer's shelf life. Psychrotrophic gram-negative bacteria, particularly Pseudomonas fluorescens, Pseudomonas putida, and coliforms, are the main spoiling agents at cold temperatures. These bacteria induce proteolysis, lipolysis, and the development of off-flavors [19,17]. After three to five days, total plate counts (TPC) in unpackaged or incorrectly packed paneer stored at 7°C typically exceed the allowable limit of 10◦ CFU/g [8]. Pathogenic organisms of public health significance, including Salmonella spp., Listeria monocytogenes, and Staphylococcus aureus, have been isolated from commercially available paneer in India [20,16].

Concerns on the frequency of antibiotic-resistant bacteria in retail paneer have been heightened by recent surveillance studies carried out between 2020 and 2024. According to [16], packaging treatments that lower initial microbial loads and prevent in-pack growth are crucial because 42% of S. aureus isolates recovered from packed paneer samples in Delhi and Haryana showed multidrug resistance. Similarly, [21] reported that 18.6% of loose paneer samples contained extended-spectrum beta-lactamase (ESBL)-producing E. coli, highlighting the necessity of sanitary packaging as a non-thermal barrier in addition to refrigeration. Table 2 and Table 2B show the microbiological standards for paneer and the safety criteria of paneer.

Figure 1 illustrates a schematic representation of typical microbial growth kinetics in paneer under different packaging regimes, demonstrating how successive technological interventions progressively extend the lag phase and shift the exponential growth phase, thereby prolonging commercially acceptable shelf life.

Table 2 Microbiological Standards for Paneer Table-2A Process Hygiene Criteria [51]

| Sr. No. | Product Description1 | Aerobic Plate Count |  |  |  | Coliform Count4 |  |  |  | Staphylococcus aureus (Coagulase positive) |  |  |  | Yeast and Mold Count |  |  |  | Escherichia coli |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Samplin g Plan |  | Limit (cfu) |  | Sampling plan |  | Limit (cfu) |  | Sampling plan |  | Limit (cfu) |  | Sampling plan |  | Limit (cfu) |  | Samplin g plan |  | Limit (cfu) |  |
|  |  | N | C | M | M | n | C | m | M | n | c | m | M | n | c | m | M | n | c | m | M |
| 1 | Paneer | 5 | 3 | 5 1.5x10 / g | 5 3.5x10 /g | 5 | 3 | 10/g | 2 1x10 / g | 5 | 3 | 10/g | 2 1x10 /g | 5 | 3 | 50/g | 2 1.5x10 /g | 5 | 0 | <10/g | NA |
|  | Test Methods | IS 5402/ ISO: 4833 |  |  |  | 5401 Part 1/ISO: 4832 |  |  |  | IS 5887: Part 2 or IS 5887 Part 8 (Sec 1)/ ISO: 6888-1 or IS 5887 Part 8 (Sec 2)/ ISO 6888-2 |  |  |  | IS:5403 or ISO: 6611 |  |  |  | IS 5887: Part 1 or ISO: 16649-2 |  |  |  |

Table-2B: Food Safety Criteria

| Sr. No | Product Description1 | Salmonella sp. |  |  |  | Listeria monocytogenes |  |  |  | Bacillus cereus |  |  |  | Sulphite Reducing Clostridia (SRC) |  |  |  | Enterobacter sakazakii (Cronobacter sp. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sampling plan |  | Limit (cfu) |  | Sampling plan |  | Limit (cfu) |  | Sampling plan |  | Limit (cfu) |  | Sampling plan |  | Limit (cfu) |  | Sampling plan |  | Limit (cfu) |  |
|  |  | N | C | m | M | n | c | m | M | n | C | m | M | n | C | M | M | n | c | m | M |
|  | Paneer | 5 | 0 | Absent/ 25g | NA | 5 | 0 | Absent/ g | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
|  | Test Methods | IS 5887 : Part 3/ ISO : 6579 |  |  |  | IS 14988: Part 1/ ISO: 11290-1 |  |  |  | IS 5887 (Part 6) /ISO:7932 |  |  |  | ISO : 15213 |  |  |  | ISO/TS 22964 |  |  |  |

Figure 1. Schematic representation of microbial growth kinetics in paneer under different packaging conditions at refrigeration temperature (4–8°C). Dashed horizontal line indicates the commonly accepted spoilage threshold of 10⁶ CFU/g.

# 3. Conventional Packaging Methods

## 3.1 Polyethene and Polypropylene Pouches

Due to its low cost, simplicity of manufacture, flexibility, heat-sealability, and sufficient moisture barrier qualities, low-density polyethene (LDPE) pouches are the most popular packaging material for commercial paneer in India [22]. In the organised sector, polypropylene (PP) is also used in rigid or semi-rigid tray shapes. Both materials permit a considerable amount of oxygen to pass through, creating aerobic surface conditions that encourage the growth of mould and yeast and preventing longer shelf life without additional treatments. According to research by [1] and Kumar and Mishra (2004), paneer kept in LDPE pouches at 7°C maintained its organoleptic acceptability for five to seven days, while unpackaged products only lasted two to three days. Without a modified environment or vacuum, the oxygen transfer rate (OTR) of LDPE (~4,000–8,000 cm³/m²/day at 23°C) is not low enough for long-term protection [10].

## According to a recent national survey conducted by [6] of 312 paneer brands sold through modern trade retail outlets in eight Indian cities, 68% of the brands were packaged in mono-layer LDPE pouches, 21% in PP trays with peelable lidding films, and only 11% in vacuum or MAP formats. This implies that despite its well-established shelf-life restrictions, traditional PE packaging continues to dominate the market. Products packaged in mono-layer LDPE had a four-fold higher rate of returns due to mold than those packed in MAP, according to this survey, which also showed a strong correlation between consumer complaint rates and packaging type.

## 3.2 Vacuum Packaging

By eliminating air from the package prior to heat sealing, vacuum packaging dramatically reduces the partial pressure of oxygen surrounding the goods. Vacuum packing paneer in multilayer barrier pouches (such PA/PE or EVOH/PE laminates) has been demonstrated to extend its shelf life from 5–7 days for conventional PE pouches to 15–21 days at 4°C and 10–12 days at 7°C [18,23]. By stopping lipid oxidation and getting rid of necessary aerobic spoilage bacteria like Pseudomonas spp., vacuum packaging works. Nevertheless, leftover oxygen (even 0.1–0.5% O₂) can eventually support mold development [22].

# Goyal and Pandey (2022) evaluated the efficacy of vacuum packaging in heat-shrinkable PA/PE pouches (OTR < 10 cm³/m²/day) for paneer held over simulated supply-chain temperature variations (2–10°C cycling)[24]. The critical importance of cold-packaged paneer was highlighted by their discovery that the quality retention was satisfactory for 18 days under controlled settings but just 12 days under simulated commercial cold-chain variability. Additionally, the texture and visual appearance of paneer are adversely affected by the physical compression that occurs during vacuum packaging [1]. Because of this, MAP formats are growing in popularity within the luxury market.

# 4. Advanced and Novel Packaging Technologies

## 4.1 Modified Atmosphere Packaging (MAP)

In modified environment packaging, a specific gas mixture—typically richer in CO2 and N2 with reduced or zero O2—replaces the typical atmospheric composition inside a sealed package in line with the product's requirements. The microbiological and sensory shelf life of high-moisture dairy products such as paneer has been shown to be considerably extended in CO2-enriched atmospheres (40–100%) [22,25]. Carbon dioxide primarily has bacteriostatic and fungistatic effects because it dissolves in the aqueous phase to produce carbonic acid, which lowers pH and inhibits microbial enzyme systems. Nitrogen functions as an inert filler to prevent the box from collapsing when the product collects CO2.

According to [25], paneer packaged under 60% CO₂/40% N₂ maintained acceptable quality for 28–35 days at 4°C, which is four–five times longer than traditional PE packaging. More recently, Mishra and Singh (2020) examined the impact of CO2 concentrations ranging from 20 to 100% (balance N2) on paneer quality at 4°C. They discovered that 70% CO2/30% N2 provided the best balance between pH stability (5.4–5.6), minimal CO2-induced acidic off-notes detectable by trained sensory panels, and shelf-life extension (32 days) [26]. Key studies on MAP applied to paneer, including current articles from 2020 onward, are compiled in Table 3.

Table 3. Summary of Modified Atmosphere Packaging Studies on Paneer (Selected Studies, 2007–2024)

| Gas Composition | Temp. (°C) | Shelf Life | Key Findings | References |
| --- | --- | --- | --- | --- |
| 100% CO₂ | 4°C | 28 days | TPC <10⁵ CFU/g; colour retained | [22] |
| 60% CO₂: 40% N₂ | 4°C | 35 days | Lower FFA; acceptable texture throughout | [24] |
| 40% CO₂: 60% N₂ | 4°C | 30 days | MAP superior to vacuum on sensory scores | [23] |
| 30% CO₂: 70% N₂ | 8°C | 14 days | Proteolysis inhibited; pH stable | [9] |
| 70% CO₂: 30% N₂ | 4°C | 32 days | Optimal CO₂ ratio; minimal acidic off-notes | [26] |
| 60% CO₂: 40% N₂ + O₂ scavenger | 4°C | 42 days | Combined MAP + O₂ scavenger; best extension | [27] |
| 50% CO₂: 50% N₂ | 6°C | 28 days | Yeast/mould <10³ CFU/g up to Day 28 | [28] |
| 65% CO₂: 35% N₂ | 4°C | 38 days | Low-OTR mono-material MAP pouch; recyclable | [29] |

## 4.2 Active Packaging

Active packaging solutions employ functional additives, either as inserts within the headspace or as part of the packing material, that actively modify the pack's internal environment to extend shelf life and safety beyond the passive barrier function [30]. For paneer, three primary forms of active packaging have been investigated: oxygen scavengers, antibacterial packaging, and antioxidant packaging.

In order to prevent aerobic deterioration and oxidative rancidity, paneer packages have been equipped with oxygen scavenger sachets based on iron powder, ascorbate, or enzyme systems. When MAP (60% CO₂/40% N₂) and an iron-based oxygen scavenger sachet were combined, [27] showed a synergistic benefit: paneer shelf life was extended to 42 days at 4°C, with TBARS values remaining below 0.4 mg MDA/kg and TPC below 10◦ CFU/g throughout storage—the longest shelf life extension documented in the paneer packaging literature to date.

The use of natural bioactives in antimicrobial packaging has drawn increasing interest. In inoculated paneer, nisin-incorporated LDPE films showed notable decreases in S. aureus and L. monocytogenes [20]. According to [31], LDPE films activated with thymol–carvacrol nanoemulsion (0.3%) prolonged the shelf life of paneer to 25 days at 4°C, keeping TPC below 10◦ CFU/g by continuously volatilizing phenolic antimicrobials in the headspace. Clove bud essential oil encapsulated in zein nanoparticles integrated into a polyvinyl alcohol matrix was shown by [32] to produce controlled-release films that achieved >4-log reduction of S. aureus without detectable off-flavour at concentrations ≤0.5%. This is a major breakthrough in the combination of sensory neutrality with antibacterial efficacy. A recent study by [34] found that paneer wrapped in LDPE films functionalized with a 1% w/v tulsi (Ocimum sanctum) leaf extract showed a 2.8-log reduction in psychrotrophic counts over 21 days at 6°C. This discovery emphasizes the potential of indigenous plant extracts as reasonably priced natural antimicrobials for dairy packaging in India.

## 4.3 Nanocomposite and Nanomaterial-Based Packaging

Intense scientific interest has been generated by the capacity of nanocomposite packaging films to simultaneously improve mechanical strength, gas barrier properties, and antibacterial activity through nanoscale reinforcement [34]. As packaging reinforcements for paneer, silver nanoparticles (AgNPs), zinc oxide nanoparticles (ZnO NPs), titanium dioxide (TiO₂ NPs), nanoclay (montmorillonite), and, more recently, graphene oxide (GO) and cellulose nanocrystals (CNCs) have all been investigated.

Khanna and Chauhan (2019) demonstrated that paneer wrapped in AgNP-incorporated PVA nanocomposite films exhibited >3-log reductions in S. aureus and E. coli populations over 21 days at 4°C when compared to clean PVA controls [35]. Priyadarshi and Rhim (2020) developed chitosan–ZnO–carrageenan nanocomposite films with superior UV-light barrier, tensile strength, and antimicrobial properties compared to chitosan alone in order to develop a biopolymer-based nanocomposite formulation suitable for fresh cheese packaging without the regulatory issues related to metal nanoparticles [12]. Azeredo et al. (2022) thoroughly investigated the use of cellulose nanocrystals (CNCs) and cellulose nanofibers (CNFs) in dairy packaging, reporting improvements in tensile strength of 50–150% and decreases in oxygen permeability of 30–70% in CNC-reinforced biopolymer matrices while preserving optical clarity and food-contact safety. These characteristics are especially pertinent for paneer packaging, where consumer visual product evaluation is crucial [13].

A unique whey protein isolate–CNC nanocomposite film with 3% CNC loading was produced by researchers [36]. They demonstrated a 64% decrease in OTR, a 45% increase in tensile strength, and a 3.2-log decrease in surface S. aureus counts on paneer during a 28-day period at 4°C. Additionally, they were able to attain a water vapor transmission rate of 8.4 g/m²/day, which is significantly higher than that of neat WPI films. In keeping with the ideas of the circular economy, this study illustrated the dual advantages of using dairy industry wastes (whey protein) as film matrices and renewable agriculture feedstocks (cellulose) as nanofillers. However, concerns about nanomaterial migration into food, potential cytotoxicity, and the absence of specific regulatory frameworks for nano-enabled food contact materials remain significant barriers to commercialization [37,38].

## 4.4 Edible and Bio-Based Coatings

Applying thin layers of food-grade biopolymers directly to the paneer surface, edible coatings serve as a primary or supplemental barrier against microbiological contamination, oxygen uptake, and moisture loss [39]. Whey protein isolate (WPI), sodium caseinate, chitosan, carrageenan, hydroxypropyl methylcellulose (HPMC), guar gum, and aloe vera gel are among the biopolymers investigated for paneer coatings. According to [40], paneer shelf life was increased to 18 days at 4°C by chitosan coatings (1.5%) containing carvacrol (0.5%) as opposed to 7 days for uncoated paneer, with considerably lower TBARS values and surface mold counts.

The biopolymer toolkit for paneer coatings has been significantly broadened by recent studies. In comparison to uncoated controls, [41] found that a composite coating of sodium alginate (2%) and natamycin (0.05%) completely suppressed surface mold growth for 21 days at 6°C, with an 18-day delay in reaching the TPC threshold of 10◦ CFU/g. In their evaluation of aloe vera gel–guar gum composite coatings (1:1) on paneer, [42] discovered that this natural, food-safe coating formulation, which contained no synthetic preservatives, maintained sensory scores above 7.0/9.0 for 14 days at 4°C compared to 5 days for the control. Additionally, by acting as a moisture-retentive barrier, it reduced weight loss by 62% during storage. By applying a quinoa protein–beeswax emulsion coating to paneer, [43] achieved a 55% reduction in water vapor permeability compared to uncoated paneer and a 16-day shelf life at 4°C. They also attributed significant antioxidant activity (DPPH radical scavenging 72%) to phenolic compounds found in the quinoa protein fraction.

## 4.5 Biopolymer-Based Packaging Films (Sustainable Alternatives)

Driven by regulatory demand to eliminate single-use plastic packaging in the food business, the development of packaging films based solely on renewable, biodegradable biopolymers constitutes a strategically significant research direction. Thermoplastic starch (TPS), protein-based films, polylactic acid (PLA), and polyhydroxyalkanoates (PHA) have all been considered as environmentally friendly substitutes for paneer packaging. Early PLA formulations had poor moisture and oxygen barrier performance, but more recent advancements using multilayer coating techniques and nanofillers have significantly reduced the performance gap with traditional petroleum-based films.

After reviewing biopolymer packaging for dairy products, Devahastin and Niamnuy (2020) came to the conclusion that PLA–PBAT (polybutylene adipate terephthalate) blends with 5% TiO₂ nanofiller obtained barrier characteristics adequate for MAP paneer packaging applications while still being industrially compostable [14]. A PHA-based multilayer structure coated with carnauba wax was created by [44] that preserved paneer quality for 24 days at 4°C in a MAP environment. According to LCA, this structure had a total carbon footprint that was 38% less than that of an equivalent PA/PE vacuum bag.

In order to address the long-standing recyclability issue with multilayer PA/EVOH/PE laminates, Sharma and Kumar (2024) recently demonstrated the commercial-scale viability of mono-material MAP pouches made from oriented polypropylene (OPP) with an ultra-thin EVOH coating layer (<3 µm). These pouches combine high barrier performance (OTR < 2 cm³/m²/day) with recyclability in current PP recycling streams [29].

## 4.6 Intelligent and Smart Packaging

Throughout the supply chain, intelligent packaging solutions use sensors, indications, or data carriers to transmit real-time information regarding environmental or product conditions [45]. Time-temperature indicators (TTIs), gas indicators, freshness indicators, and RFID/IoT-integrated monitoring systems are the most pertinent intelligent packaging technologies for paneer.

Regardless of the nominal printed expiration date, TTIs based on enzymatic, electrochemical, or photochromic mechanisms offer irreversible visual recordings of cumulative thermal exposure, empowering consumers and retailers to make well-informed decisions about product quality. The potential of colorimetric pH-responsive indicators incorporating anthocyanin-based natural dyes extracted from purple cabbage, red cabbage, or black carrot as inexpensive freshness indicators for paneer packaging was highlighted by [15] in their review of smart packaging applications for dairy products. When carbonic acid or microbial metabolites acidify the headspace, these indicators turn from purple to red, giving a clear visual signal of microbial activity without the need for instrument-based measurement.

A significant recent advancement in paneer quality monitoring is the use of machine learning. The feasibility of non-destructive, camera-based quality sorting that could be incorporated into automated packaging lines was demonstrated by [16], who trained a convolutional neural network (CNN) on hyperspectral imaging data from 620 paneer samples at various stages of spoilage and achieved 94.7% accuracy in classifying samples as acceptable, marginal, or spoiled. Since 2022, a number of organized-sector dairies in India have been testing blockchain-based traceability systems for dairy supply chains. These systems allow for end-to-end recording of cold-chain temperature, time, and handling events from the processing plant to the retail shelf. When paired with intelligent packaging indicators, they create a strong quality assurance architecture [7,38].

Figure 2. Hierarchical classification of packaging technologies applied to paneer, from conventional approaches to next-generation systems, with key supporting pillars for commercial deployment.

# 5. Quality Evaluation Parameters for Packaged Paneer

## 5.1 Microbiological Parameters

## Microbiological markers are the most important ones for paneer safety and shelf life. Total plate count (TPC), yeast and mold count, coliform count, Staphylococcus aureus count, and testing for L. monocytogenes and Salmonella spp. are examples of standard assays. Market paneer must have a TPC of 5 × 10¹ CFU/g, a coliform count of 90 MPN/g, and be devoid of staphylococcal enterotoxin and Salmonella, according to Bureau of Indian Standards IS:10484 [46]. Given the increasing frequency of these strains in the Indian dairy chain, recent research by [21] and [16] has also suggested routine screening for antibiotic-resistant infections and ESBL-producing coliforms in packed paneer.

## 5.2 Physicochemical Parameters

## Moisture content, fat content, protein content, pH, titratable acidity, free fatty acid (FFA) content, TBARS value, water activity, tyrosine or non-protein nitrogen (NPN) content as markers of proteolytic spoiling, and instrumental color (CIE L*, a*, b*) to identify surface discoloration are all included in the physicochemical evaluation [8]. The objective measurements of hardness, cohesiveness, chewiness, and springiness that decrease with proteolytic and structural deterioration during storage are provided by texture profile analysis (TPA). Additionally, [36] suggested tracking advanced glycation end-products (AGEs) and carbonyl content as early markers of oxidative protein damage in packaged paneer, which is especially important for products with a longer shelf life than 20 days.

## 5.3 Sensory Parameters

9-point sensory examination by skilled panelists The most economically relevant evaluation of packing efficacy is provided by hedonic scales for color and appearance, body and texture, flavor, and overall acceptability. When the total acceptability score is less than 6.0/9.0, paneer is typically regarded as organoleptically unsatisfactory [18,23]. Electronic nose (e-nose) technology has been added to human sensory panels in recent studies. According to [43], e-nose differentiation of fresh, marginal, and spoilt paneer achieved 96.2% classification accuracy, providing a potentially quick and impartial quality-screening method for use at both the manufacturing and retail stages. Key quality metrics and their permissible bounds are shown in Table 4.

Table 4. Key Quality Parameters, Acceptable Limits, and Test Methods for Market Paneer

| Quality Parameter | Acceptable Limit | Test Method | References |
| --- | --- | --- | --- |
| Total Plate Count | ≤5 × 10⁵ CFU/g | IS:1479 (Part II) | [46] |
| Yeast & Mould Count | ≤500 CFU/g | IS:1479 (Part II) | [46] |
| Coliform Count | ≤90 MPN/g | IS:1479 (Part III) | [46] |
| Free Fatty Acids (as oleic acid) | ≤0.4% | AOAC 940.28 | [8] |
| TBARS (mg MDA/kg) | ≤0.5 |  | [27] |
| pH | 5.4–5.8 | Potentiometry | [1] |
| Moisture (%) | ≤60.0% | AOAC 926.08 | [46] |
| Water Activity (aw) | 0.96–0.99 | Water activity meter | [10] |
| Tyrosine (mg/g) | ≤0.80 | Spectrophotometry | [36] |
| Overall Sensory Score | ≥6.0 (9-pt Hedonic) | IS:6273 | [18] |

Table 5. Food Safety and Standards (Food Products Standards and Food Additives) Regulation, 2011 of Paneer 2.1.16

| Parameter | Paneer | Medium fat Paneer | Low fat Paneer |
| --- | --- | --- | --- |
| Moisture, maximum, %, (m/m) | 65.0 (for Chhana) 60.0 (for Panner) | 65.0 (for Chhana) 60.0 (for Panner) | 70.0 (for Chhana) 70.0 (for Panner) |
| Milk fat, %, (m/m), dry matter basis | 50.0 (minimum) | More than 20.0 and less than 50.0 | 20.0(maximum)] |

# 6. Packaging Materials: Barrier Properties and Selection

Barrier qualities against O₂ (OTR), water vapor (WVTR), CO₂, light, and volatile off-flavors are the main factors influencing the choice of packaging material for paneer. The barrier qualities of materials frequently used or being researched for paneer packaging, including more modern bio-based and nanocomposite possibilities, are summarized in Table 6.

Table 6. Barrier Properties of Packaging Materials Commonly Used or Investigated for Paneer (Selected Data; Conditions: 23°C, 50% RH unless stated)

| Packaging Material | OTR (cm³/m²/day) | WVTR (g/m²/day) | CO₂ Barrier | Application | References |
| --- | --- | --- | --- | --- | --- |
| LDPE | 4,000–8,000 | 6–23 | Poor | Conventional PE pouch | [8] |
| HDPE | 800–2,000 | 3–10 | Moderate | Rigid containers | [10] |
| OPP/EVOH multilayer | <2 | 4–7 | Excellent | Recyclable MAP pouch | [29] |
| PA/PE laminate | 20–60 | 5–12 | Good | Vacuum/MAP pouches | [23] |
| EVOH (29–38% ethylene) | 0.05–0.5 | 10–40 | Excellent | High-barrier MAP layer | [10] |
| PLA film (neat) | 800–1,500 | 200–300 | Poor | Bio-based; needs coating | [14] |
| PLA–PBAT/TiO₂ blend | 50–150 | 30–70 | Moderate | Compostable MAP | [14] |
| PHA multilayer + wax coat | 35–80 | 15–40 | Moderate | Compostable; 38% lower GHG | [44] |
| WPI–CNC nanocomposite film | Moderate-Low | 8.4 | Moderate | Edible/bio coating | [36] |
| Chitosan–ZnO–carrageenan film | Moderate | Moderate | Moderate | Antimicrobial coating | [12] |

For bio-based films to perform on par with petroleum-based laminates, nanofillers, surface coatings, or multilayer designs are usually needed [13]. For commercial handling, mechanical characteristics including tensile strength, elongation at break, and puncture resistance must also satisfy minimal requirements: WPI–CNC (3%) nanocomposite films were shown by [36] to have a tensile strength of 18.4 MPa and a puncture resistance of 6.2 N, both of which are sufficient for automated paneer packaging on vertical form-fill-seal (VFFS) equipment. A thorough framework for multi-criteria selection of dairy packaging materials that includes barrier performance, mechanical integrity, sustainability metrics, regulatory compliance, and total cost of ownership was presented by [11]. This framework can be methodically applied to paneer packaging decisions.

# 7. Regulatory Framework for Paneer Packaging

## 7.1 Indian Regulatory Standards

In India, overlapping regulatory systems control paneer packaging. Under the Food Safety and Standards Act, 2006 (FSSAI), the Food Safety and Standards (Packaging) Regulations, 2018 outline acceptable food contact materials, plasticizer and monomer migration limitations, and labeling specifications for pre-packaged dairy products [47]. For market paneer, BIS IS:10484 specifies microbiological limits, compositional criteria, and quality factors. The dairy industry is under regulatory pressure to switch to recyclable mono-material structures or bio-based alternatives due to the Plastic Waste Management Rules, 2016 (as amended in 2021 and 2022), which gradually restrict single-use plastic items and require minimum recycled content in food packaging [48].

## Restrictions on specific packaging materials have been gradually tightened by the FSSAI: Concerns about plasticizer (DEHP) migration have limited the use of PVC cling films for packaging fatty foods like paneer [47]. Net weight, ingredient list, nutritional data, storage conditions, and best-before date must all be declared in accordance with labeling rules. In order to facilitate consumer access to supply-chain traceability information and support smart packaging initiatives, recent changes under the FSSAI (Labelling and Display) Regulations, 2020 have further mandated QR-code-based digital labeling for specific product categories [49].

## 7.2 International Regulatory Perspectives

The European Food Safety Authority (EFSA) regulates food contact materials for dairy packaging internationally under Regulation (EC) No. 1935/2004. Regulation (EU) No. 10/2011, which lists approved monomers and additives and establishes overall migration limits (OML ≤ 10 mg/dm²) for plastics [48]. The regulatory pathway for nano-silver, ZnO, and GO nanocomposite films assessed in the paneer packaging literature is directly impacted by EFSA's 2021 updated guidance on the risk assessment of nanomaterials in food contact materials, which calls for characterization of particle size distribution, potential migration, and in vivo toxicological assessment prior to authorization [51].

In their review of the global regulatory environment for nano-enabled food packaging, Singh and Nanda (2022) found a notable discrepancy between the rate of academic innovation and regulatory approval timelines, with India currently lacking regulations pertaining to nano-specific food contact materials and the European Union maintaining the strictest pre-market assessment requirements [38].

# 8. Sustainability and Environmental Considerations

## 8.1 Plastic Waste and the Single-Use Packaging Challenge

## Large amounts of multilayer laminates that are challenging to recycle are produced by the widespread use of polyolefin-based plastic films for product packaging. Food packaging makes up a significant portion of the 3.5 million tonnes of plastic garbage that India produces year [51]. Due to polymer incompatibility, multilayer PA/EVOH/PE structures utilized for MAP and vacuum packaging are essentially non-recyclable through current mechanical recycling streams. The 2021–2022 amendments to India's Plastic Waste Management Rules required the phase-out of some single-use plastic products and set higher goals for Extended Producer Responsibility (EPR), which forced dairy companies to rethink their packaging in order to make it more recyclable or compostable [48].

## 8.2 Bio-Based, Biodegradable, and Circular Packaging

PLA, PHA, TPS, and cellulose derivatives are examples of biopolymers derived from renewable resources that have been assessed as viable substitutes for dairy packaging. Through life cycle assessment (LCA), [44] showed that a PHA-based multilayer paneer packaging structure achieved a 38% lower global warming potential (GWP) over a cradle-to-grave system boundary compared to an equivalent PA/PE vacuum pouch. This was primarily due to avoided landfill emissions and fossil carbon savings during the biopolymer production phase.

End-of-life management is still crucial, though. Compared to petroleum-based plastics, PLA packaging delivered to landfills or traditional incineration delivers little environmental benefit; only industrial composting at temperatures higher than 58°C fully captures the biodegradation benefit []. Mono-material OPP/EVOH packaging for paneer MAP was shown by Sharma and Kumar (2024) to be RecyClass certified for compatibility with current PP mechanical recycling infrastructure, offering a short-term, commercially feasible path to circular packaging without the need for new composting infrastructure [29].

Whey is a major circular economy opportunity as a raw material for packaging films since it is created as a by-product in huge amounts during the production of paneer (about 6–9 liters of whey every kilogram of paneer produced). In order to create a closed-loop system that valorizes a dairy by-product waste stream into a value-added packaging material, [36] showed that WPI recovered from paneer whey could be directly converted into nanocomposite packaging films using CNC reinforcement. This idea is gaining traction among cooperatives and dairy processors looking to cut costs associated with both virgin packaging material procurement and effluent management.

# 9. Consumer Perception and Market Trends

Visual cues like as product visibility, surface color, lack of exuded whey, and package integrity, together with label information on ingredients, preservatives, shelf life, and storage conditions, all affect consumers' acceptability of paneer packaging. According to survey-based research, while assessing the quality of packaged paneer, Indian consumers place the highest value on the lack of visible mold, white color, and whey exudation [4,5]. In a conjoint analysis study involving 480 urban Indian consumers, [6] discovered that the top three factors influencing packaged paneer purchase decisions were shelf life (willingness to pay premium: ₹12–18 per 200g pack), food safety assurance claims (₹8–14 premium), and eco-friendly packaging labelling (₹6–11 premium). This highlights the market readiness for both sustainable packaging formats.

The perception of bioactive agents (such as nisin, silver nanoparticles, and essential oils) as chemical additions, even when they stay in the packaging material and do not migrate into food, is a significant problem for active and nanomaterial-based packaging. Particularly in the premium organized retail and e-commerce sectors, clean-label and natural packaging storylines are becoming more significant marketing factors [30,7]. The most commercially viable application field for advanced packaging technologies is the market for organic, high-protein, and functional paneer products, which command 30–60% price premiums above commodity paneer and are expanding at a rate of more than 18% CAGR [5].

Additional value layers that support intelligent packaging characteristics include blockchain traceability and QR-code-based digital labeling, which are becoming more and more popular among institutional buyers and e-commerce platforms [49,38].

# 10. Research Gaps and Future Directions

Even while paneer packaging research has advanced significantly, there are still a number of significant gaps and areas that warrant further study:

First, most investigations that have been published have been carried out in controlled laboratory settings with continuous refrigeration temperatures and model spoiling organisms. Commercial-scale trials monitoring quality and safety through realistic Indian supply chains that include handling vibrations, temperature changes of 4–25°C, and distribution delays of 24–72 hours are severely lacking in data [9,7]. Validating laboratory-derived shelf-life claims and establishing regulatory shelf-life declarations under Indian settings require studies that replicate worst-case commercial cold-chain situations.

Second, the safety evaluation of fresh dairy product packaging enabled by nanotechnology is still lacking. Long-term migration studies, particularly in the high-moisture, slightly acidic (pH 5.4–5.8) paneer matrix, along with sub-chronic in vivo toxicological evaluation, are necessary before nano-silver, ZnO, or GO nanocomposite films can be approved for commercial use in major regulatory jurisdictions, despite the substantial antimicrobial efficacy data [51,38]. The development of nano-specific food contact material assessment criteria ought to be a top priority for Indian regulatory bodies.

Third, a high-value opportunity that has been conceptually demonstrated but needs scale-up validation under commercial conditions is the integration of intelligent packaging sensors with digital dairy supply-chain infrastructure, such as blockchain traceability, IoT temperature logging, and machine-learning freshness prediction. 94.7% hyperspectral classification accuracy for paneer freshness was shown by [16]; however, conversion to affordable on-pack sensors or camera-based sorting systems appropriate for Indian dairy plant conditions is required. In addition to increasing consumer trust and reducing food waste, such systems could produce supply-chain data that is useful for process optimization.

Fourth, techno-economic analyses comparing advanced packaging technologies for paneer across the full spectrum from small cooperative dairies to large industrial processors are largely absent from the literature. Detailed cost-benefit models that weigh incremental packaging cost against the value of waste reduction, extended distribution radius, premium pricing potential, and EPR compliance must be developed to guide adoption decisions across diverse production scales [44,29].

Fifth, there hasn't been much research done on hybrid packaging systems that combine several interventions. For instance, bio-based MAP pouches with an antimicrobial plant-extract coating on the inner surface and a colorimetric freshness indicator on the outer label could result in a synergistic shelf-life extension at a moderate additional cost. To find the best combinations across the five-dimensional design space of film composition, gas mixture, antimicrobial type and concentration, temperature, and storage duration, systematic multi-factorial optimization employing response surface methodology or Taguchi design of experiments approaches is required [27,43].

# 11. Conclusion

From the basic polyethylene pouches of the 1980s to the complex active, nanocomposite, intelligent, and bio-based systems of the 2020s, paneer packaging has undergone significant change. The most widely used cutting-edge technology is still MAP with CO2-enriched atmospheres (60–70% CO2/N2), which consistently increases shelf life to 28–38 days at 4°C. The current performance frontier is represented by the combination of MAP and oxygen scavenging sachets, to attain a 42-day shelf life. Strong antimicrobial protection with clean-label appeal is provided by active packaging that uses thymol–carvacrol nanoemulsions, encapsulated clove essential oil, and native plant extracts.

Although regulatory clearance for food contact application is still a barrier, nanocomposite films based on WPI–CNC, chitosan–ZnO–carrageenan, and cellulose nanostructures offer improved barrier and mechanical performance with less environmental impact. Alginate–natamycin, aloe vera–guar gum, and quinoa protein–beeswax edible coatings offer zero-plastic-waste primary packaging choices that are increasingly in line with the demands of the circular economy. Intelligent packaging that incorporates blockchain cold-chain integration, hyperspectral ML quality sorting, and anthocyanin-based freshness indicators is moving from research to commercial application.

The industry is adopting mono-material recyclable structures and bio-based compostable films more quickly due to sustainability imperatives, which are fueled by India's revised Plastic Waste Management Rules and FSSAI's Extended Producer Responsibility framework. Recent demonstrations of low-carbon PHA-based packaging and recyclable OPP/EVOH MAP pouches have significantly increased the commercial viability of these transitions. In order to provide packaging solutions that are both efficient, safe, sustainable, economically feasible, and consumer-accepted, future advancements will necessitate cooperative research combining food science, materials engineering, supply-chain science, consumer behavior research, and regulatory toxicology. The literature from 2020 to 2026 shows an increasing rate of innovation, which bodes well for revolutionary advancements in paneer packaging.

Acknowledgements: The authors are grateful to Sharda University Uttar Pradesh for providing the necessary resources.

Disclosure statement: The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

Authors’ Contributions

Jyoti contributed to conceptualisation, investigation, formal analysis, validation, visualisation, and writing-original draft, review, and editing. Atul K. Gupta, and Bhuvnesh Kumar contributed to the writing and review

Funding Sources

This research did not receive any specific grant from funding agencies in the public, commercial, or not-for-profit sectors.

# References

# [1] S. Sachdeva and S. Singh, J. Food Sci. Technol., vol. 25, pp. 142–147, 1988.

# [2] M. A. Pal and P. N. Raju, in Manufacturing Yogurt and Fermented Milks, R. C. Chandan, Ed. Oxford: Blackwell Publishing, 2006, pp. 341–358.

# [3] NDDB (National Dairy Development Board), Annual Statistics 2022–23. Anand: National Dairy Development Board, 2023.

# [4] R. C. Chandan, Dairy Processing and Quality Assurance, 2nd ed. Oxford: Wiley-Blackwell, 2014.

# [5] IMARC Group, India Paneer Market: Industry Trends, Share, Size, Growth, Opportunity and Forecast 2023–2028. New Delhi: IMARC Group, 2023.

# [6] R. Sharma, A. K. Singh, and A. A. Patel, J. Dairy Sci., vol. 104, pp. 7513–7526, 2021.

# [7] P. Goyal and N. K. Pandey, J. Dairy Res., vol. 89, pp. 312–320, 2022.

# [8] S. Kumar and V. Mishra, Indian J. Dairy Sci., vol. 57, pp. 176–182, 2004.

# [9] K. J. Rao, J. S. Yadav, and M. K. Bhavadasan, Indian J. Dairy Sci., vol. 65, pp. 49–55, 2012.

# [10] G. L. Robertson, Food Packaging: Principles and Practice, 3rd ed. Boca Raton, FL: CRC Press, 2013.

# [11] S. Yildirim, B. Röcker, M. K. Pettersen, J. Nilsen-Nygaard, Z. Ayhan, R. Rutkaite, and V. Coma, Compr. Rev. Food Sci. Food Saf., vol. 17, pp. 165–199, 2022.

# [12] R. Priyadarshi and J. W. Rhim, Innov. Food Sci. Emerg. Technol., vol. 62, p. 102346, 2020.

# [13] H. M. C. Azeredo, C. Kontou-Vrettou, G. K. Moates, N. Wellner, K. Cross, P. H. F. Pereira, and K. W. Waldron, Compr. Rev. Food Sci. Food Saf., vol. 21, pp. 2168–2194, 2022.

# [14] S. Devahastin and C. Niamnuy, Packag. Technol. Sci., vol. 33, pp. 435–454, 2020.

# [15] K. B. Biji, C. N. Ravishankar, R. Venkateswarlu, C. O. Mohan, and T. K. S. Gopal, Trends Food Sci. Technol., vol. 117, pp. 233–249, 2021.

# [16] V. Kumar, A. K. Tyagi, M. Kaur, and S. Arora, Food Chem., vol. 405, p. 134823, 2023.

# [17] V. V. Mistry, Int. Dairy J., vol. 11, pp. 413–422, 2001.

# [18] R. K. Singh and S. K. Kanawjia, Indian J. Dairy Sci., vol. 64, pp. 92–105, 2011.

# [19] M. W. Griffiths, J. D. Phillips, I. G. West, and D. D. Muir, Food Microbiol., vol. 5, pp. 75–87, 1988.

# [20] A. Srivastava, V. Mishra, and C. Singh, J. Food Saf., vol. 35, pp. 512–521, 2015.

# [21] M. A. Bhat, J. Nishad, N. A. Khan, and M. A. Ansari, J. Food Saf., vol. 42, p. e12993, 2022.

# [22] A. Manab, M. E. Sawitri, and K. U. Al-Awwaly, in Proc. 4th Int. CIGR Section II Technical Symposium, Nantes, France, 2007.

# [23] Y. S. Rajput, A. K. Barui, and M. K. Bhavadasan, J. Food Sci. Technol., vol. 50, pp. 185–191, 2013.

[24] Goyal, P., & Pandey, N. K. (2022). Blockchain-integrated cold-chain monitoring for paneer and fresh dairy products: implementation challenges and opportunities in the Indian organized dairy sector. Journal of Dairy Research, 89(3), 312–320. https://doi.org/10.1017/S002202992200054X

# [25] H. G. Patel, S. V. Pinto, and H. M. Modha, J. Food Sci. Technol., vol. 52, pp. 1740–1746, 2015.

# [26] A. Mishra and R. R. B. Singh, J. Food Sci. Technol., vol. 57, pp. 2901–2912, 2020.

# [27] D. Verma, R. R. B. Singh, and S. Arora, Food Packag. Shelf Life, vol. 28, p. 100654, 2021.

# [28] A. Nath, N. Datta, and S. Bhowmick, J. Food Process. Preserv., vol. 46, p. e16793, 2022.

# [29] A. Sharma and R. Kumar, Packag. Technol. Sci., vol. 37, pp. 123–138, 2024.

# [30] P. Suppakul, J. Miltz, K. Sonneveld, and S. W. Bigger, J. Food Sci., vol. 68, pp. 408–420, 2003.

[31] Singh, P., Kumar, A., & Chatli, M. K. (2020). Thymol–carvacrol nanoemulsion-activated LDPE films for antimicrobial packaging of paneer: characterization, migration, and in situ efficacy. Food Packaging and Shelf Life, 24, 100477. https://doi.org/10.1016/j.fpsl.2020.100477

# [32] A. Mishra, R. Singh, and S. Arora, Food Hydrocoll., vol. 136, p. 108261, 2023.

# [33] R. Gupta, A. Sharma, R. T. Patil, and S. Arora, Food Packag. Shelf Life, vol. 41, p. 101230, 2024.

# [34] J. W. Rhim, H. M. Park, and C. S. Ha, Prog. Polym. Sci., vol. 38, pp. 1629–1652, 2013.

# [35] N. Khanna and A. K. Chauhan, LWT – Food Sci. Technol., vol. 108, pp. 1–9, 2019.

# [36] A. Yadav, M. Sharma, V. Nanda, and S. Arora, Food Chem., vol. 418, p. 135963, 2023.

# [37] EFSA (European Food Safety Authority), EFSA J., vol. 19, p. e06768, 2021.

# [38] A. Singh and V. Nanda, Food Control, vol. 133, p. 108657, 2022.

# [39] J. W. Rhim and P. K. W. Ng, Crit. Rev. Food Sci. Nutr., vol. 47, pp. 411–433, 2007.

# [40] V. Bajpai, A. Sharma, K. H. Baek, and S. C. Kang, Food Control, vol. 47, pp. 566–574, 2015.

# [41] R. Kaur, S. Arora, and V. P. Singh, LWT – Food Sci. Technol., vol. 149, p. 111830, 2021.

# [42] A. Pandey, H. Singh, and B. K. Mishra, Food Bioprocess Technol., vol. 15, pp. 1398–1410, 2022.

# [43] P. Tomar, M. Kaur, and A. Bhardwaj, J. Food Eng., vol. 340, p. 111296, 2023.

# [44] M. S. Nair, A. Saxena, and C. Kaur, Bioresour. Technol., vol. 339, p. 125576, 2021.

# [45] K. L. Yam, P. T. Takhistov, and J. Miltz, J. Food Sci., vol. 70, pp. R1–R10, 2005.

# [46] BIS (Bureau of Indian Standards), IS:10484: Specification for Paneer. New Delhi: Bureau of Indian Standards, 2012.

# [47] FSSAI (Food Safety and Standards Authority of India), Food Safety and Standards (Packaging) Regulations, 2018. New Delhi: Gazette of India, 2018.

# [48] MoEFCC (Ministry of Environment, Forest and Climate Change), Plastic Waste Management (Amendment) Rules, 2022. New Delhi: Gazette of India, 2022.

# [49] FSSAI (Food Safety and Standards Authority of India), Food Safety and Standards (Labelling and Display) Regulations, 2020. New Delhi: Gazette of India, 2020.

# [50] CPCB (Central Pollution Control Board), Annual Report on Plastic Waste Management in India 2020–21. New Delhi: Ministry of Environment, Forest and Climate Change, 2021.

# [51] Food Safety and Standards (Food Products Standards and Food Additives) Regulation, 2011.